Ultrasonic sensor

Working:

This sensor works by emitting a high-frequency sound waves that humans can not here.

1.frequency used 40hz

2.works on sonar principal

3.common modal used for projects HC-SR04

40hz frequency will be abel to travel well in air and reflect from a solid object.

main logic of working:

- Sensor sends 8 ultrasonic pulses from TRIG

- Pulses hit an object

- Echo is reflected back

- ECHO pin goes HIGH for the time taken by sound to travel to object + back

- Distance is calculated using time of flight

pin and it connection:

| pin | name | connect to |
| --- | --- | --- |
| 1 | VCC | +5v supply |
| 2 | TRIG | digital pin of any microcontroller) |
| 3 | ECHO | any digital pin (when we use ESP32 we have to use voltage divider 5v to 3.3v) |
| 4 | GND | ground pin of microcontroller |

distance calculation formula:

speed of sound in air = 343m/s

formula

distance =(time * speed of sound) /2

formula using in microcontrollers

distance(cm) = time / 58

Range and operating:

operating voltage = 5v

measuring range = 2cm to 400cm

accuracy = +or - 3mm

measuring angle = 15degree

Application:

1.obstacale avoidance robot

2.water level monitoring

3.parking sensor

4.distance measuring device

5.smart dustbin

6.drone altitude sensing

image of ultrasonic sensor: